PRODUCT REQUIREMENTS DOCUMENT

(PRD)

EcoPatch

A Biodegradable, Zero-Waste IoT Health Monitoring System for the Elderly

# 0. Document Info

| Field | Content |
| --- | --- |
| Product name | EcoPatch — a two-part IoT health monitoring system (biodegradable disposable patch + reusable smart core) |
| Problem addressed | Continuous vital-sign monitoring for an aging population, combined with reducing medical e-waste |
| Document version | v1.0 |
| Date drafted | August 16, 2026 |
| Status | Draft — pending review |
| Author | Trí Max |
| Related scope | Layer 1 (biodegradable patch), Layer 2 (nRF52832 IoT core), Layer 3 (mobile app + cloud AI) |

# 1. Background & Problem

Two global crises are converging: an aging population and growing e-waste. Elderly people face silent risks of stroke and dehydration and need continuous vital-sign monitoring, yet current wearables are often bulky, uncomfortable, and irritating to fragile aging skin. At the same time, single-use medical devices (sensor patches, electrode tapes, etc.) generate millions of tons of plastic and toxic heavy metal waste every year.

EcoPatch addresses both problems at once with a two-part IoT health monitoring system: the skin-contact part is a fully biodegradable patch, while the electronics live in a compact, detachable, reusable smart core.

# 2. The Solution

## 2.1 Disposable Biodegradable Patch

- Made from Bacterial Cellulose, harvested through fermentation of aged coconut water — repurposing an agricultural byproduct common in Vietnam.

- The conductive circuit is printed with Bio-Carbon ink pyrolyzed from spent coffee grounds, forming flexible printed electronics.

- Soft as a second skin; fully composts into 100% organic fertilizer within 7-14 days.

## 2.2 Reusable Smart Core

- A tiny, detachable IoT module that physically connects to the patch via magnets/pogo pins.

- Reads biosignals (ECG, GSR) and transmits data over Bluetooth Low Energy.

- Rechargeable, designed to last for years — a one-time purchase.

# 3. Target Users

| Persona | Primary need | Interaction with EcoPatch |
| --- | --- | --- |
| Elderly person (wearer) | Comfort, no skin irritation, minimal complexity | Wears the patch on the skin; replaces it every 7-14 days |
| Family member / Caregiver | Early warning of stroke/dehydration risk, peace of mind when not nearby | Receives emergency push notifications via the mobile app when anomalies are detected |
| Medical center / Doctor (optional) | Track the patient's long-term vital-sign trends | Views time-series data via integration/API (if shared) |
| System operator/admin | Manage subscriptions, ensure the IoT core operates reliably | Manages accounts and monthly patch subscription orders via the app/admin console |

# 4. Scope

## 4.1 In scope

- Biodegradable patch materials & design: Bacterial Cellulose substrate, Bio-Carbon conductive ink, skin-safe biomedical adhesive layer.

- IoT core firmware (nRF52832 BLE): reads ECG and GSR sensors, processes signals, transmits data over BLE 5.0.

- The detachable physical connection mechanism between the smart core and the patch (magnets/pogo pins).

- Mobile app (React Native): displays real-time health metrics.

- Cloud AI: time-series analysis to predict and alert on stroke/dehydration risk.

- Razor-and-Blade business model: one-time IoT core purchase + monthly patch box subscription.

## 4.2 Out of scope

- Large-scale industrial manufacturing process for the biodegradable patch (a separate R&D/manufacturing workstream).

- Medical device certification (FDA/CE) — requires a separate regulatory roadmap, outside this technical PRD.

- Direct integration with hospital electronic health record (EHR) systems (future roadmap item).

# 5. System Architecture

The system has three layers: biodegradable hardware in contact with the skin, a reusable IoT core that processes signals, and a mobile/cloud AI software layer that analyzes and alerts.

## 5.1 Layer 1 — Biodegradable Patch (Materials/Hardware)

- Substrate: Bacterial Cellulose — a biological coconut-jelly-like membrane with an ultra-durable 3D nanofiber structure, good moisture retention, and natural adhesion.

- Circuit: printed with Bio-Carbon ink from pyrolyzed spent coffee grounds, blended with a biological binder.

- A skin-safe biomedical adhesive layer for fragile elderly skin, allowing the skin to "breathe".

## 5.2 Layer 2 — Reusable IoT Core (Firmware)

- Signal-processing chip nRF52832 (BLE) — ECG sensor, GSR sensor.

- Analyzes heart rate and hydration level (electrolytes via sweat).

- Physically connects to the patch via magnets/pogo pins; transmits data over Bluetooth Low Energy 5.0.

## 5.3 Layer 3 — Mobile App & Cloud AI (Software)

- React Native App: displays real-time health metrics for the user/family.

- Cloud AI: a time-series AI model that predicts and alerts on stroke/dehydration risk before it happens.

# 6. Key Features

| Feature | Description |
| --- | --- |
| 24/7 vital-sign monitoring | Basic ECG, heart rate (HR), and sweat electrolyte analysis to warn of dehydration |
| Real-time IoT alerts | Anomaly-detection algorithm that sends emergency notifications to family members' phones or a medical center |
| Compostable | The skin-contact patch can be removed and buried in soil, fully decomposing into plant nutrients |
| Highly biocompatible | The coconut-cellulose membrane cools the skin and lets it "breathe", avoiding the redness/itching common with standard medical adhesives |

# 7. Functional Requirements

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-01 | The IoT core must continuously read ECG and GSR signals from the patch through the magnet/pogo-pin connection | High |
| FR-02 | The IoT core must transmit vital-sign data to the mobile app over BLE 5.0 with latency acceptable for near-real-time monitoring | High |
| FR-03 | The mobile app must display heart rate, basic ECG, and hydration level in real time | High |
| FR-04 | Cloud AI must run time-series analysis to detect anomalies and predict stroke/dehydration risk | High |
| FR-05 | The system must send an emergency push notification to family members/a medical center when a serious anomaly is detected | High |
| FR-06 | The system must support detaching and reattaching the IoT core to a new patch without losing configuration data | High |
| FR-07 | The app must support subscription management (ordering, renewal) for the monthly patch box | Medium |
| FR-08 | The system must warn the user when the patch is nearing the end of its lifespan (7-14 days) so it can be replaced | Medium |
| FR-09 | The IoT core must support recharging and display battery status | Medium |
| FR-10 | The system must store historical vital-sign data to support long-term trend viewing | Medium |
| FR-11 | The system should support sharing data with doctors/medical centers via API or export | Low |

# 8. Non-Functional Requirements

| Category | Requirement |
| --- | --- |
| Biocompatibility | The adhesive and patch must be absolutely safe for fragile/sensitive elderly skin, with no irritation |
| Biodegradability | The patch must fully decompose into organic fertilizer within 7-14 days after being buried in soil |
| IoT core battery life | Must last at least the full lifespan of one patch (7-14 days) between charges |
| Alert latency | Emergency notifications must reach family members within seconds to minutes of an anomaly being detected |
| Data security | Vital-sign data transmitted over BLE and stored in the cloud must be encrypted and comply with medical data protection regulations |
| Mechanical connector durability | The magnet/pogo-pin mechanism must withstand repeated attach/detach cycles (at least matching the expected annual subscription cycle count) |
| Sensor extensibility | The IoT core architecture should allow adding new sensor types in the future (e.g., SpO2, skin temperature) |

# 9. Materials & Printing Technology

| Component | Description |
| --- | --- |
| Bacterial Cellulose (BC) | Harvested from fermented aged coconut water (a Vietnamese agricultural byproduct); an ultra-durable 3D nanofiber structure with good moisture retention and natural adhesion |
| Conductive Coffee Ink | Spent coffee grounds pyrolyzed into conductive carbon (Bio-carbon), blended with a biological binder to form flexible printed-circuit ink |

# 10. Business Model

EcoPatch applies a "Razor and Blade" model combined with a Circular Economy approach:

- One-time purchase (IoT Core): customers buy a rechargeable smart chip core used for years.

- Monthly subscription: sells boxes of biodegradable patches (30 pieces/month), creating a sustainable recurring revenue stream.

# 11. Success Metrics

| Metric | Target |
| --- | --- |
| Full decomposition time of the patch | 7-14 days after being buried in soil, matching the Zero-Waste commitment |
| User skin irritation rate | As low as possible — tracked via user feedback/clinical trials |
| Stroke/dehydration risk prediction accuracy | Maximize sensitivity to minimize missed real alerts |
| False alarm rate | Minimize to avoid causing unnecessary anxiety for family members |
| Monthly subscription renewal rate | A key recurring-revenue health metric for the business model |
| Average number of reuse cycles per IoT core | Higher is better — reflects durability and value of the upfront investment |

# 12. Risks & Assumptions

## 12.1 Risks

- Bio-Carbon ink from coffee grounds may have less stable conductivity than traditional metal-based ink, affecting signal accuracy.

- Patch decomposition speed may vary with soil/climate conditions, making the 7-14 day target hard to guarantee in every case.

- Repeated attach/detach of the magnet/pogo-pin connector may degrade over time, causing intermittent signal loss.

- Medical device certification requirements may extend time-to-market and add significant regulatory cost.

## 12.2 Assumptions

- Elderly users own a smartphone or have a family member who can help operate the app.

- The supply of spent coffee grounds and aged coconut water is stable enough to support large-scale production in Vietnam.

- Users are willing to pay a monthly subscription in exchange for peace of mind and convenience.

# 13. Proposed Roadmap

| Phase | Content |
| --- | --- |
| Completed / Concept | Defined materials (BC + Bio-Carbon ink), the 3-layer architecture, and the Razor-and-Blade business model |
| Short term | Build patch + IoT core prototypes; validate conductivity and skin safety (internal testing) |
| Medium term | Small-scale clinical trials with real elderly users; optimize the stroke/dehydration alert AI model |
| Long term | Pursue medical device certification; expand sensor set (SpO2, skin temperature); integrate with healthcare systems/EHR |

# 14. Open Questions

- Has the conductivity and durability of the Bio-Carbon ink been validated with experimental data?

- What mechanism ensures the patch doesn't accidentally peel off during an elderly person's daily activities?

- What is the optimal subscription pricing strategy to balance recurring revenue with affordability for lower-income elderly users?

- What regulatory/medical certification roadmap is required before commercialization in Vietnam and other markets?